Poseidon X2-Denitrator

Instructions

Congratulations on your purchase! Your denitrator will provide many years of reliable service when used properly. Please read these instructions carefully before installing your denitrator.

Contents:

1.  Denitrator canisters with plastic inserts filled with X-2 Denitrification media

2.  Tubing 3ft of each; “IN” tubing (white), and “OUT” tubing (blue) with regulator

valve (blue handle) attached

3.  Pump with clear vinyl tubing adapter pieces attached

4.  Opening wrench tool

5.  Mounting bracket

Instructions:

ATTACH TUBING

•  Attach white and blue hoses to denitrator by inserting the tubing into the corresponding

fitting at the top of each canister.  The left side (first canister) marked with “IN” (for

white tubing) and the right side (second canister) marked “OUT” (for blue tubing).

ATTACH PUMP

•  Attach the pump to the white tubing by inserting the other end of the

white tubing into clear adapter vinyl tubing on pump.

•  Set pump to maximum flow and submerge in sump. Please verify pump is completely

under water.  Make sure pump does not pick up sediment residue that sometimes

accumulates at the bottom of sump areas. Sediment will enter the media chamber and

clog some of the media prohibiting the anaerobic denitrification process. Clean sump

debris prior to installation of pump.

PLACEMENT:

IN SUMP / WET DRY

•  The pump sits in the sump, feeding water into the denitrator.  Water exits back into the

sump, preferably away from the pump.

NO SUMP / WET DRY

•   For those who do not have a sump, the pump may be placed inside the aquarium while

the canister must be mounted or placed beside, above, or behind the tank nearby or

level to the pump.  Place the pump on one side of the aquarium and the blue return line

at the other end of the aquarium for optimal filtration.

Poseidon X2-Denitrator

Instructions

Page 2

PROCEDURE

•  Turn on the pump until canister fills with aquarium water and water begins to trickle

out of the blue line.  The regulator VALVE must be fully OPEN at this time.  Once

water comes trickling out of the blue line, CLOSE the regulator valve, and shut the

pump off leaving water inside the canister.  Aquarium water should remain in the

canister for 24 hours in order to break in the build up of anaerobic bacteria.

RUN THE FILTER

•  After a period of 24 hours, turn on the pump and OPEN the regulator valve to produce

a very slow and steady trickle coming out from the blue tubing.  The amount coming

out can be a constant fast paced trickle to a very slow stream, (not to exceed 50 gallons

per hour).  Continued use will filter nitrates from the water. After 1-2 weeks of use,

nitrate concentration should start to fall. As long as nitrate concentration levels remain

in control, the media does not have to be replaced.

•  Monitor the flow coming out from the regulator valve once in a while to ensure proper

flow rate.  If at times the flow rate appears to be slower than other times, this is normal

as denitrification is occurring.  If there appears to be no flow rate, increase the valve

for adjustment.

The Denitrator should maintain LOW to NO Nitrate levels for many months (average of 8-12months) before the denitrification media pack needs replacement.  For systems with initial nitrate concentrations in excess of 80ppm, the denitrator may take up to one month to notice any significant change in nitrate reduction.

Aquariums with initial nitrate concentrations in excess of 80ppm should conduct water exchanges prior to installing the denitrator.  Bring the concentration level to about 20ppm or less, before hooking up the denitrator.  If the denitrator is installed with initial nitrate concentration levels in excess of 80ppm, it will take much longer for the initial reduction process to occur.

For questions regarding your denitrator please email: PoseidonWarehouse@yahoo.com

Replacement media and other related supplies can be conveniently purchased at:

www.PoseidonWarehouse.com