Poseidon X-3 DeNitrator

Instructions

Congratulations on your purchase! Your denitrator will provide many years of reliable service when used properly.  Please read these instructions carefully before installing your DeNitrator.

Contents:

1.  Denitrator blue canisters with plastic inserts filled with X-3

Denitrification media pack

2.  Two 4ft Tube sections:  “IN” tubing (white) with control valve attached,

“OUT” tubing (blue)

3.  Pump with clear tubing adapter pieces inserted

4.  Opening wrench tool

5.  Mounting bracket

Instructions:

ATTACH TUBING

•  Attach white and blue hoses to the denitrator by inserting the tubing into

the corresponding fitting at the top of each canister.  The left side (first

canister) marked with “IN” (for white tubing) and the right side (third

canister) marked “OUT” (for blue tubing).

ATTACH PUMP

•  The Pump comes already attached to the white tubing which has a

control valve near the "IN" port of the unit.

•  Place the pump inside the sump in an area that is sediment free and

completely under water.  Clean the sump from any debris accumulation

prior to installing the pump. If you do not have a sump, place the pump

inside the aquarium and make sure it is completely underwater. Position

the pump so that it draws water from one end of the tank via  (white line)

and returns it to the other end of the tank via (blue line).

PLACEMENT OF THE UNIT:

IN SUMP / WET DRY

•  The DeNitrator unit may be mounted inside a cabinet or just resting on

the cabinet floor nearby the sump.

NO SUMP / WET DRY

•The DeNitrator may be mounted behind or nearby the top of the

aquarium with the pump inside the tank feeding the unit via (white line).

You may also place the unit on top of a canopy or mount the unit onto

the canopy.

PROCEDURE

•  With the pump fully submerged in the water, and the control valve on the

white line fully open, turn on the pump by

plugging it in.

•  When all three canisters are full and water is coming out from the blue

line, restrict the control valve so that water trickles out slowly.  Some

irregular water flow may occur at this time while the air gets purged out

of the canisters.

•  To adjust the flow rate, close off the control valve and allow the water to

flow at approximately 5mL per 50 seconds. Take a 5mL testing tube and

have the blue line drip into it filling it up to the 5mL mark in pproximately

50 sec. The system can now be left running at this flow rate.  Aenarobic

conditions will soon build up within the canisters at this flow rate and the

unit will begin to lower nitrate levels.

•  Monitor the flow coming out from the blue line from time to time to

ensure the proper flow rate.  Adjust the control valve on the white line if

necessary to maintain the 5mL/50sec flow rate.

The Denitrator should maintain LOW to NO Nitrate levels for many months (average of 10-12months) before the denitrification media pack needs replacement.  For systems with initial nitrate concentrations in excess of 80ppm, the denitrator may take up to one month to notice any significant change in nitrate reduction.  Aquariums with initial nitrate concentrations in excess of 80ppm should conduct water exchanges prior to installing the denitrator.  Bring the concentration level to about 20ppm or less, before hooking up the denitrator. Water changes may still be performed during the operation of the DeNitrator.

Replacement media and many other supplies and products can be conveniently purchased at:  www.PoseidonWarehouse.com